Powerful BRUSHLESS Motor Driver

Instruction Manual

The instructions in this manual are valid for the following versions:

V4.2

Firmware Version:

V3.0

Table of contents

Hardware and Wiring Guide

Device Operating Modes

I2C Command Overview

Control Algorithm Basics

Tuning

Known Issues

Hardware and Wiring Guide

The motor driver has a input voltage range of 6-25V. The motor driver will not work reliably if the input voltage is under 6V and if the input voltage exceeds 25V the motor driver is likely to be damaged. Damage which could occur due to overvoltage include broken input voltage sensing functionality and exploded capacitors. Since high current is expected at the input, using 20AWG (0.5mm2) wires are recommended. Thinner wires may overheat when the motor is under high load, and thicker wires may not fit in the screw terminals.

The maximum continuous output current of the motor driver is 5A (Peak of sine wave) or 3.5A (RMS). The motor driver can tolerate 8A (Peak of sine wave) or 5.6A (RMS) for short periods not exceeding 3 seconds. If the continuous current limit is exceeded the motor driver will shut down due to overheating when the temperature reaches 175C (typical value but can be anywhere between 165C and 185C). If there is a short circuit on the output or some other short term overcurrent condition exceeding 16A (typical value but can be anywhere between 10A and 22A) the motor driver will also shut down.

The I2C port of the motor driver uses 3.3V logic and has weak pullups to 3.3V internally. Please do not connect it to 5V logic devices such as Arduino, due to the risk of damaging the I2C port. To achieve the highest performance two external pullup resistors should be connected on SCL and SDA pins. The other side of the resistors must be connected to 3.3V. The recommended resistor value range is between 1KΩ and 2.2KΩ. The maximum I2C speed supported by the motor driver is 1MHz (Fast Mode Plus).

The motor sensor port provides a 3.3V output with a maximum output current is 5mA. If this is exceeded, the 3.3V rail voltage will decrease significantly. Only Sin/Cos encoders are supported currently. More encoder types may be added in the future (such as magnetic analog or PWM, Incremental quadrature). Differential signalling is not supported. Hall sensors are not supported.

The motor driver has many exposed parts which must not touch any metal objects (such as loose wires) when the motor driver is powered on, otherwise the motor driver is likely to be damaged.

The motor driver can be mounted on a 3d printed holder. There are two half cut holes, centres 24mm apart, which fit M3 screws that secure the motor driver to the mount. Leave a gap to allow airflow along the bottom of the motor driver, to improve the thermal performance and reduce the chances of overheating. PCB outline is 24x40mm When designing the holder, leave 0.1mm tolerance around the outline.

Device Operating Modes

This page lists the control modes supported by the hardware. Software support is not yet available for some modes. If you would like to use a mode that is not implemented in software, please contact support.

Trapezoidal Control (No plans to implement)

Voltage Command Mode: Input command is proportional to the average voltage applied to the motor. (No plans to implement)

Sinusoidal Control

Voltage Command Mode: Input command is proportional to the average voltage applied to the motor. (Implemented)

Field Oriented Control (Implemented)

Torque Command Mode: Input command is proportional to the torque of the motor. (Implemented)

All Control Modes:

Speed Command Mode: Motor driver tries to regulate the motor speed to be the requested speed regardless of the load on the motor. Sometimes the requested speed may not be attainable due to too much load on the motor or insufficient input voltage. (Implemented)

Position Command Mode: Motor driver tries to move the motor to the position specified. There may be some oscillations while the motor settles into the final position. (Implemented)

Position Sensor Support

Sin/Cos Encoder

Incremental Encoder (No plans to implement)

Magnetic Encoder Analog or PWM (No plans to implement)

Sensorless (No plans to implement)

I2C Command Overview

Send the command (uint8_t), followed by the command data to the motor driver, using the i2c address of the motor driver. You can either perform an i2c scan (with only the motor driver connected to the i2c bus) or read the label on the motor driver to find out the i2c address.

To get responses from the motor driver, request the expected number of bytes from the motor driver immediately after sending the command. If there was no previous command needing a response, and a request for data is received, the motor driver will respond with the Quick Data Readout (defined later). In case multiple commands needing a response are sent but without reading the response in between, the earlier responses will be discarded and only the latest response will be kept. Once this response is read, further reads will be replied to using the Quick Data Readout.

The command data and responses are listed in the order they are transmitted. For multibyte values such as uint32_t and float (IEEE 754), they need to be transmitted Least Significant Byte first, and Most Significant Bit first (Little Endian). See below table for example.

| Byte | First byte is Least Significant Byte |  |  |  |  |  |  |  | … | … | Last Byte is Most Significant Byte |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 (LSB) | … | … | 231 (MSB) | 230 | 229 | 228 | 227 | 226 | 225 | 224 |

| Command | Command Data | Description |
| --- | --- | --- |
| 0x00 | uint32_t version | Get Version. Responds with the version. |
| 0x01 | None | Clear Faults. Clears overcurrent/overtemperature faults on motor driver to restart operation. Motor control signals reset. No response. |
| 0x10 | int32_t SIGNAL | Send Voltage Control Signal. Direction is also part of this signal. 0 = 0V, 3399 = supply voltage. Negative voltages allowed. No response. |
| 0x11 | int32_t SIGNAL | Send Torque Control Signal. Direction is also part of this signal. 1LSB = 2-16A No response. |
| 0x12 | int32_t SIGNAL | Send Speed Control Signal. Direction is also part of this signal. 1LSB = 2-16POS/second No response. |
| 0x13 | uint32_t POSITION uint8_t ELECANGLE | Send Position Control Signal. Motor takes shortest path towards POSITION. ELECANGLE gives finer control over motor position. No response. |
| 0x20 | uint8_t CONFIG | Change Operating Mode and Sensor Type. Motor must be stopped before sending this command. Add the Operating Mode and Sensor type to get CONFIG. No response. Operating Mode; Sensor Type. 0x01 Trapezoidal; 0x10 Sin/Cos Encoder. 0x02 Sinusoidal; 0x20 Incremental Encoder. 0x03 Field Oriented (FOC); 0x30 Sensorless. 0x0F Calibration Mode. |
| 0x21 | uint8_t CONFIG | Change Command Mode. No response. TODO: allow changing this on the fly. Command Mode. 0x01 Voltage (Trapeziodal/Sinusoidal). 0x02 Torque (FOC). 0x0C Speed (All Operating Modes). 0x0D Position (All Operating Modes). 0x0F Calibration Mode. |
| 0x22 | uint8_t FORMAT | Change Quick Data Readout Format. See below for FORMAT number. Default 0x00. No response. |
| 0x30 | uint32_t ELECANGLEOFFSET | Change ELECANGLEOFFSET calibration value for the encoder. Motor should be stopped before sending this command. No response. |
| 0x31 | int32_t EAOPERSPEED | Change ELECANGLEOFFSET/(POS/second) calibration value for Trapezoidal Mode and Sinusoidal Mode. No response. |
| 0x32 | int32_t SINCOSCENTRE | Change centre calibration value of sin/cos encoder, between 0 and 4095. 0 = 0V, 4095 = 3.3V. No response. |
| 0x33 | int32_t currentlimit | Change Current Limit (Only active in FOC mode), between 0 (0A) and 524288 (8A). No response. |
| 0x34 | int32_t speedlimit | Change Speed Limit (Only active in Speed or Position mode), between 0 and 546133333. 1LSB = 2^-16 POS/second. No response. |
| 0x38 | uint8_t startstop | Start/stop Calibration. Send 1 to start and 0 to stop. Motor driver must be configured in calibration mode before sending calibration commands. No response. |
| 0x39 | uint8_t calibration_state, uint32_t ELECANGLEOFFSET, int32_t SINCOSCENTRE | Get Calibration Status. Only works in calibration mode. Responds with calibration_state, ELECANGLEOFFSET, SINCOSCENTRE. calibration_state: 0: calibration not started, between 1 and 249: calibration running, 254: calibration error, 255: calibration success. ELECANGLEOFFSET and SINCOSCENTRE are only valid when calibration_state = 255. |
| 0x3A | uint32_t voltage, int32_t speed, uint32_t scycles, uint32_t cycles | Set Calibration Options, only used during calibration. No response. Voltage: 0 = 0V, 6798 = supply voltage. Speed: 1LSB = 2-16POS/second. Scycles: Settling time in PWM cycles. Cycles: Calibration time in cycles. 50000 cycles = 1 second. Defaults: Voltage 250, Speed 1048576, Scycles 50000, Cycles 500000. |
| 0x40 | int32_t kp, int32_t ki | Change PID tuning constants for IQ in FOC modes. Range between 0 and 1024. Recommended to stop motor before sending this command. No response. |
| 0x41 | int32_t kp, int32_t ki | Change PID tuning constants for ID in FOC modes. Range between 0 and 1024. Recommended to stop motor before sending this command. No response. |
| 0x42 | float kp, float ki, float kd | Change PID tuning constants for Speed Control. Recommended to stop motor before sending this command. No response. |
| 0x43 | float kp, float ki, float kd | Change PID tuning constants for Position Control. Recommended to stop motor before sending this command. No response. |
| 0x44 | float pos_region_boundary | Change boundary between linear and sqrt for the position PID controller. Boundary position proportional to pos_region_boundary^2. 65536 = 1 POS. No response. |
| ??? | int32_t D_current | Todo: Implement D current control in Torque mode in case user wants to do flux weakening |

Quick Data Readout Short Format (FORMAT = 0x00)

| Byte | Data |
| --- | --- |
| 0-3 | uint32_t POSITION. 1LSB = 1 full electrical rotation of motor. |
| 4-7 | int32_t SPEED. 1LSB = 2-16POS/second |
| 8 | ERROR1 Byte, see below for details |
| 9 | ERROR2 Byte, see below for details |

ERROR1 Byte

| Bit LSB = 0 MSB = 7 | Data |
| --- | --- |
| 0 | Motor Driver Miscellaneous Error. This bit will be 1 if there is any error in the motor driver, except configuration error. |
| 1 | Configuration Error. This bit will be 1 if the configuration is wrong. |
| 2 | Command Error. This bit will be 1 if an unrecognised command is sent. |
| 3 | Timing Error. This bit will be 1 if the motor driver CPU is overloaded, causing timing constraints to be violated. |
| 4-7 | Reserved for future use. |

ERROR2 Byte (Not implemented yet)

| Bit LSB = 0 MSB = 7 | Data |
| --- | --- |
| 0 | Overcurrent. This bit will be 1 if overcurrent is detected on low side MOSFET in phase U. |
| 1 | Overcurrent. This bit will be 1 if overcurrent is detected on high side MOSFET in phase U. |
| 2 | Overcurrent. This bit will be 1 if overcurrent is detected on low side MOSFET in phase V. |
| 3 | Overcurrent. This bit will be 1 if overcurrent is detected on high side MOSFET in phase V. |
| 4 | Overcurrent. This bit will be 1 if overcurrent is detected on low side MOSFET in phase W. |
| 5 | Overcurrent. This bit will be 1 if overcurrent is detected on high side MOSFET in phase W. |
| 6 | Overtemperature Shutdown. This bit will be 1 if motor driver internal temperature exceeds 180 degc. |
| 7 | Overtemperature Warning. This bit will be 1 if motor driver internal temperature exceeds 145 degc. |

Control Algorithm Basics

Space Vector PWM:

The motor driver uses Space Vector PWM (in Sinusoidal and FOC modes) to generate the three PWM signals fed to the motor. The motor driver uses Space Vector PWM with one phase always at GND, to reduce MOSFET switching losses. The Space Vector PWM algorithm is tightly integrated with the lowside current sensors, enabling 100% duty cycle operation.

Sinusoidal Control:

Sinusoidal control uses the Space Vector PWM to apply a voltage 90 degrees offset to the electrical angle of the motor. At high speeds, the current will not be in phase with the voltage, so the EAOPERSPEED parameter increases the voltage offset angle proportional to the speed, so the current will be in phase with the voltage. Sinusoidal Control does not use the current sensors, and if the motor is blocked or the control signal ramps too quickly it is easy to cause an overcurrent in the motor driver. Therefore, this mode is not recommended for normal use and is only intended for debugging when FOC is not working.

When using Sinusoidal Control, the control signal represents the peak voltage of the output. The RMS voltage is equal to 1/sqrt(2) or 0.707 times the peak voltage.

Field Oriented Control (FOC):

FOC uses the current sensors and the Park and Clarke mathematical transformations to convert the phase currents to currents in the D and Q reference frame, where the D axis is aligned with the magnetic field of the rotor’s permanent magnets, and the Q axis is perpendicular to the D axis. Field Oriented Control uses PID to adjust the voltage sent to the motor, so that the current in the D and Q axis is regulated to the desired value.

When using FOC, the control signal represents the peak current of the output. The RMS current is equal to 1/sqrt(2) or 0.707 times the peak current.

Regenerative Braking:

The motor driver supports regenerative braking while in FOC mode. It is possible in sinusoidal mode but will not work well. The motor driver must be powered by either a rechargeable battery or a power supply which will tolerate reverse current. For power supplies which do not tolerate reverse current, you can put a large Transient Voltage Suppression diode (TVS diode) with suitable ratings in parallel with the power supply to absorb the reverse current. The regenerative braking current limit is the same as the normal current limit. The maximum reverse current seen by the power supply is the same as the regenerative braking current (at full speed), and your power supply needs to tolerate this reverse current, otherwise reduce the current limit accordingly.

To activate regenerative braking, in FOC mode simply set the Q current the reverse direction to the motor velocity. It will be automatically activated in Speed and Position modes when using FOC.

Speed Control:

Speed control uses PID to control either the voltage (Sinusoidal / Trapezoidal Mode) or the current (FOC mode).

The unit used for speed control is POS per second, where POS is electrical revolutions. To convert electrical revolutions to physical revolutions, you need to divide it by the number of pole pairs in your motor (find it in the motor datasheet). If your motor has a gearbox, you also need to divide the result by the gear ratio to get the physical revolutions.

Position Control:

The motor driver uses a constant acceleration model position control algorithm. This method of position control allows very fast motor movements with minimal overshoot. Due to sensor noise being significant when the motor is close to the final position and moving at low speed, the position control algorithm automatically transitions into linear mode for low speeds to prevent oscillations.

In constant acceleration model mode, the speed of the motor is proportional to the square root of the distance remaining and capped at the speed limit. In linear mode the speed of the motor is proportional to the distance remaining. The speed will transition smoothly between these modes.

Tuning

Note: There may be better ways to tune the constants, so feel free to deviate from this guide if you are confident.

FOC current:

It is recommended to use the same kp and ki for the D and Q current controllers. To tune the current PID, set kp=1 and ki = 0, set a low D current and rotate the motor until the full D current is seen on one of the phases using a current clamp connected to an oscilloscope. If the D current is too small to be seen after rotating the motor, increase kp and try again.

Starting with ki = 0, increase kp until the rise time of the current is around 400us and the current is around 75% of the requested current. Increase ki slowly until the current reaches the requested value within a reasonable time. Keep increasing kp and ki until there is either overshoot or oscillations. When the maximum kp and ki values are found that do not have overshoot and oscillations, decrease both values by 25%.

ELECANGLEOFFSET and SINCOSCENTRE:

Use automatic calibration. Set a speed approximately 0.5%-2% the maximum speed of the motor, and start with a voltage setting of 500. Increase the voltage or decrease the speed until the motor spins smoothly during calibration and does not get stuck. Set the calibration cycles so that the motor sensor completes at least 10 full revolutions during the calibration. If possible, try to complete an integer amount of revolutions. Increase the calibration settling cycles so the motor fully settles at a constant velocity after this time, when changing direction during the calibration.

Once you are sure the calibration settings are correct and the motor is observed to be moving smoothly during the calibration process, the calibration results should be valid.

Speed:

Starting with ki = 0, kd = 0, increase kp until oscillations or vibrations are observed, then increase kd until the oscillations are gone. If increasing kd does not reduce the oscillations, reduce kp. Finally increase ki until there are oscillations or overshoot, then reduce ki until those are gone.

Position Control:

It is recommended to not use the I and D controller for position control and set both to 0. To tune the position PID in constant acceleration model mode, first tune FOC and speed PIDs, and attach the load to the motor. The current limit must also be set to the final value. The position PID must be re-tuned whenever the load or current limit changes.

Starting with a low value for the position kp (1e-10 recommended starting value), slowly increase kp exponentially until the motor can hit maximum speed while a moving a large distance. Then continue slowly increasing kp until there is a small but noticeable overshoot when the motor arrives at the desired position. Then slowly increase pos_region_boundary starting from 256 until any oscillations go away (make sure they are gone by trying moves of various distances). Once the oscillations are gone, decrease kp until the overshoot is gone. Finally decrease kp by 10% and increase pos_region_boundary by 25% to increase loop stability.

If constant acceleration model mode is not desired, set pos_region_boundary to 4294967296. The PID constants will be different, and you need to tune them yourself without using this guide.

Known Issues

The debug port seems to be susceptible to EMI (Electromagnetic Interference). If EMI gets into the debug port, it could cause the motor driver to behave erratically. A full power cycle is required to fix the issue. Do not connect any wires to the debug port when not in use, the wires are likely to pick up EMI and cause issues.

The I2C and sensor ports are also susceptible to EMI, but to a lesser extent. If EMI gets into the I2C port, it could cause the I2C communication to stop working, and a power cycle is required. If EMI gets into the sensor port, it could cause a temporary loss of motor control and loss of position tracking, however the issue should resolve after the EMI is removed.

If EMI problems are observed in the I2C or sensor ports, adding shielding around the cables will help. Connect the shielding to GND.

The motor driver does not start up properly if the motor is already spinning. This includes if the clear faults command is sent while the motor is spinning.