Quick Start Guide for Powerful BRUSHLESS Motor Driver:

Please note: This guide and the example code are intended for the Robomaster M2006 P36 motor. Some constants in the example code default to a value suitable for this model of motor. If using a different model of motor tuning these constants to suit the motor is required. This is not handled by the automatic calibration routine and needs to be done manually.

Please note: The motor driver only supports 3.3V communication. Do not connect to 5V logic, there is a small chance of hardware damage.

1. Connect the brushless motor to the motor driver. The current motor driver firmware cannot automatically detect the order of the motor wires, so you need to connect the black wire to PHU, yellow to PHV, red to PHW. Connect the motor sin/cos sensor cable to the sensor port.

2. Connect the I2C cable from the motor driver to the main controller. You can use raspberry pi pico, esp32, teensy, stm32, or almost any other 3.3V microcontroller with arduino ide support. Look up the default I2C pins online for your microcontroller.

3. Connect the motor driver to power. VCC is positive and GND is negative. You can use a power supply or a battery between 6V and 25V. (6S Lipo battery is OK). Please note the motor driver supports regenerative braking, so if using a power supply make sure it can handle reverse current. If it cannot, it is recommended to add a suitable TVS diode to the power supply output to prevent damage.

4. Open Arduino IDE and create a new project, and save it with your preferred project name. Find the project folder (usually in Documents/Arduino/YOURPORJECTNAME). Copy PowerfulBLDCdriver.cpp and PowerfulBLDCdriver.h into this folder. Copy calibration.ino into this folder as well.

5. Delete the YOURPORJECTNAME.ino file, and rename calibration.ino to YOURPORJECTNAME.ino. (Make sure to use your actual project name instead of YOURPORJECTNAME).

6. Connect the main microcontroller to your computer, and select the board in Arduino IDE. Look for the line "motor1.begin(25, &Wire);". You need to change the number 25 to match the I2C address of the motor driver you are using.

7. Upload the code to the board. When this step is completed the motor will start the calibration sequence.  During the calibration sequence, the motor should not have anything attached, as this will increase the error in the calibration results and could also cause the calibration to fail.

8. When programming is finished, open the Serial Monitor (under Tools menu). Once calibration is completed, the serial monitor will print out "ELECANGLEOFFSET:xxx SINCOSCENTRE::xxx". Make sure you observe the motor is spinning smoothly during and after the calibration process, to make sure the calibration was successful. Copy and paste these numbers into a notepad.

9. Now copy demo.ino into the arduino project folder. Delete the YOURPORJECTNAME.ino file, and rename demo.ino to YOURPORJECTNAME.ino. (Make sure to use your actual project name instead of YOURPORJECTNAME).

10. Look for the line "motor1.begin(25, &Wire);". You need to change the number 25 to match the I2C address of the motor driver you are using. Look for the line "motor1.setELECANGLEOFFSET(/*Paste ELECANGLEOFFSET calibration value here*/);", and replace "/*Paste ELECANGLEOFFSET calibration value here*/" with the ELECANGLEOFFSET value obtained from the calibration. Look for the line "motor1.setSINCOSCENTRE(/*Paste SINCOSCENTRE calibration value here*/);", and replace "/*Paste SINCOSCENTRE calibration value here*/" with the SINCOSCENTRE value obtained from the calibration.

11. You can upload the code to the board now, and the motor should spin quickly. You can now read the code to figure out what it does and modify it to suit your project.

12. For big number haters and people who need help on unit conversions:

Robomaster M2006 P36 has 7 pole pairs and 36:1 gearbox. To get electrical revolutions from output revolutions, you will need to multiply by 7 then multiply by 36. To get revolutions per second from revolutions per minute, divide by 60. Since 1LSB = 2^-16 electrical revolutions per second for the setSpeed command, the conversion is

Speed =  rpm * 7 * 36 / 60 / 2^-16

which can be simplified to

Speed =  rpm * 275251.2

A function to set the RPM of the Robomaster M2006 P36 is provided for convenience. Paste the function at the bottom of YOURPORJECTNAME.ino if you want to use it.

void motorsetrpm(PowerfulBLDCdriver& motor, float rpm) { motor.setSpeed(rpm * 275251.2f); }

Usage example:

motorsetrpm(motor1, 100);

Please contact support if you have trouble with following these steps, or you suspect something is wrong.