Raspberry Pi 5 Setup Guide for Powerful Brushless Motor Driver Library

- Install Raspberry Pi Imager: https://www.raspberrypi.com/software/

- Plug in your SD card into your computer. Open Raspberry Pi Imager, select device “Raspberry Pi 5”, OS “Raspberry Pi OS (64 bit)”, and your SD card. Follow the instructions to install. You may need to enter your Wi-Fi password and create a user account. You can also use your mobile phone hotspot.

Note: Use a WPA2 network on 2.4GHz or 5GHz band, otherwise you may experience compatibility issues.

- Once it finishes, put the SD card into the raspberry pi. Connect a power supply to the USB-C port. It should turn on automatically, if not, press the power button.

- Use SSH to connect to your raspberry pi. You can use this command:

ssh username@raspberrypi

Note: you may need to replace raspberrypi with the IP address of your raspberry pi if the command does not work.

- Note: Step 5 and 6 are optional, you can use SSH if you prefer.

Enter the command in the SSH session:

sudo raspi-config

Then go in and enable VNC. Reboot if required.

- Download VNC Viewer to connect to your raspberry pi and use the GUI:

https://www.realvnc.com/en/connect/download/viewer/

- Note: from this point onwards, all commands should be run on the raspberry pi.

Add dtoverlay=i2c1,pins_2_3,baudrate=1000000 to config.txt file, located in /boot/firmware folder. This changes the I2C speed on the raspberry pi to 1MHz.

- Install circuitpython by following this guide: https://learn.adafruit.com/circuitpython-on-raspberrypi-linux/installing-circuitpython-on-raspberry-pi

Note: Don’t forget to activate the virtual environment every time the raspberry pi reboots. Here’s the command as a shortcut: source env/bin/activate

- Use this command to install the motor driver library:

pip install git+https://github.com/Aw3someAndrew/SteelBar_CircuitPython_powerful_bldc_driver.git

- Download example code from https://github.com/Aw3someAndrew/SteelBar_CircuitPython_powerful_bldc_driver/tree/main/examples

Run using this command python /path/to/example/code

The example code should guide you through the motor calibration process.

# Motor Commander

Motor Commander is a new quick start example code which will help you calibrate and control your motors from the command line. It supports controlling multiple motor drivers simultaneously. It also lets you test out the different command modes of the motor driver quickly.

Before you run Motor Commander, please check the tuning constants and I2C pins are set correctly.

When you run Motor Commander, you will be asked to input the number of motor drivers and the I2C address of each. Afterwards, the motor drivers will perform a calibration and output the calibration results on screen. Once all the motors have been calibrated, the program will begin accepting commands to control the motors.

Example commands:

n2 – select motor number 2

s65536 – set speed to 65536

All commands are listed below:

| Command | Parameter | Description |
| --- | --- | --- |
| n | x | Select motor number x. All other commands operate on the selected motor. |
| m | x | Change command mode to x. See documentation for command mode descriptions. |
| p | x | Set motor position to x. |
| s | x | Set motor speed to x. |
| c | x | Set motor current limit (torque) to x. (FOC only) |
| k | x | Set position PID P constant to x. |
| b | x | Set position region boundary to x. |
| f |  | Clear faults on the motor driver |
| d | x | Delay for x milliseconds. |