Notice: Constants given with firmware version 2 are inaccurate due to the calculation bug. When migrating to version 3 please re-tune the constants or use the ones given in this page. There is a small chance of instability during deceleration when using Id Iq PID constants in the previous version, due to the constants being too small causing the current loop not responding quickly enough, leading to overcurrent protection being activated.

Note: Intended for firmware version 3

Note: The values given here are just rough estimates, they are only good enough to get the motor spinning. For the best performance it is recommended to use these as starting values and tune the PID values yourself.

Note: Sinusoidal Control is available, but FOC is recommended. Sinusoidal Control is only intended for debugging when FOC is not working properly.

Robomaster M2006 P36 Motor:

| Id Kp = 1500 | Id Ki = 200 | Iq Kp = 1500 | Iq Ki = 200 |
| --- | --- | --- | --- |
| FOC Speed Kp = 4e-2 | FOC Speed Ki = 4e-4 | FOC Speed Kd = 3e-2 |  |
| Sine Speed Kp = 4e-6 | Sine Speed Ki = 4e-7 | Sine Speed Kd = 4e-6 | EAOPERSPEED = 200000 |
| Position Kp = 275 No Load | Position Ki = 0 | Position Kd = 0 | Pos_region_boundary = 250000 |

Robomaster M3508 P19 Motor:

| Id Kp = 1600 | Id Ki = 240 | Iq Kp = 1600 | Iq Ki = 240 |
| --- | --- | --- | --- |
| FOC Speed Kp = 1e-1 | FOC Speed Ki = 1e-3 | FOC Speed Kd = 6e-2 |  |
| Sine Speed Kp = | Sine Speed Ki = | Sine Speed Kd = | EAOPERSPEED = |
| Position Kp = 150 No Load | Position Ki = 0 | Position Kd = 0 | Pos_region_boundary = 160000 |